| Code | Explanation |
| --- | --- |
| import pandas as pd | Import “pandas” data analysis package |
| import numpy as np | Import “numpy” math analysis package |
| project_name = "Project #2: Automation of NDOT Concrete Mix Design" | Defines the name of the project |
| client = "Nebraska Department of Transportation (NDOT)" | Defines the client of the project |
| cubic_yard_ft3 = 27 | Defines the number of cubic feet in one cubic yard as a variable to be used in calculation |
| unit_weight_water = 62.4 | Defines the weight of one cubic foot of water as a variable to be used in calculation |
| def water_weight_calc(A,B,C,D,E): | Begins definition of function to define weight of water required for a given amount of cementitious material |
| cementious_weight = A+B+C+D | Sums the weights of the cementitious material, as will be specified by the user |
| water_weight = cementious_weight*E | Defines the weight of the water by multiplying the weight of cement by the desired water ratio |
| return water_weight | Returns the required weight of water |
| def calc_cem_vol(A,J): | Begins definition of function to compute the volume of portland cement |
| R = A/(J*62.4) | Multiplies specific gravity of cement by volume of one cubic yard, then divides specified weight of cement by this number to get required volume of cement |
| return R | Return the required cement volume |
| def calc_fly_ash_vol(fly_ashB,fly_ashK): | Begins definition of function to compute the volume of fly ash |
| S = fly_ashB/(fly_ashK*62.4) | Multiplies specific gravity of fly ash by volume of one cubic yard, then divides specified weight of fly ash by this number to get required volume of fly ash |
| return S | Return the required fly ash volume |
| def calc_silica_fume_vol(silica_fumeC,silica_fumeL): | Begins definition of function to compute the volume of fume silica |
| T = silica_fumeC/(silica_fumeL*62.4) | Multiplies specific gravity of fume silica by volume of one cubic yard, then divides specified weight of fume silica by this number to get required volume of fume silica |
| return T | Return the required fume silica volume |
| def calc_scm_vol(scmD,scmM): | Begins definition of function to compute the volume of other cementitious material (SCM) |
| U = scmD/(scmM*62.4) | Multiplies specific gravity of SCM by volume of one cubic yard, then divides specified weight of SCM by this number to get required volume of SCM |
| return U | Return the required SCM volume |
| def calc_air_vol(air_contentF): | Begins definition of function to compute the absolute air volume, for calculation |
| V = (air_contentF/100)*27 | Converts specified air percentage into a decimal, then multiplies that by 27 to find the amount in one cubic yard |
| return V | Returns air volume |
| def calc_water_vol(water_weight_Q): | Begins definition of function to compute the absolute water volume, for calculation |
| W = water_weight_Q/62.4 | Divides specified weight of water by water weight per cubic foot to get required water volume |
| return W | Returns water volume |
| def calc_agg_vol(R,S,T,U,V,W): | Begins function to define total aggregate volume |
| X = 27 - R - S - T - U - V - W | Subtracts volume of all cementitious material, air, and water from volume of a cubic yard. The rest of this volume will be taken up by aggregates. |
| return X | Returns this remaining volume, the total aggregate volume |
| def calc_fine_agg_weight_Y(G,N,X): | Begins function to define weight of fine aggregate |
| Y = 62.4 * (G/100) * N * X | Multiplies unit weight of water, percentage of fine aggregate, the specific gravity of the fine aggregate, and the total aggregate volume to get the weight of fine aggregate |
| return Y | Returns the calculated weight of fine aggregate |
| def calc_coarse_agg_weight_Z(H,O,X): | Begins function to define weight of coarse aggregate |
| Z = 62.4 * (H/100) * O * X | Multiplies unit weight of water, percentage of coarse aggregate, the specific gravity of the fine aggregate, and the total aggregate volume to get the weight of coarse aggregate |
| return Z | Returns the calculated weight of coarse aggregate |
| def calc_other_agg_weight_AA(W,I,P,X): | Begins function to define weight of other aggregate |
| AA = W * (I/100) * P * X | Multiplies unit weight of water, percentage of other aggregate, the specific gravity of the other aggregate, and the total aggregate volume to get the weight of other aggregate |
| return AA | Returns the calculated weight of other aggregate |
| project_number = int(input("Enter project number: ")) | Defines a user input for the project name |
| concrete_class = input("Enter the class of the concrete: ") | Defines a user input for the concrete class |
| cement_A = float(input("Enter weight of cement per yard A: ")) | Defines a user input for the weight of portland cement per cubic yard |
| fly_ash_B = float(input("Enter weight of fly ash per yard B: ")) | Defines a user input for the weight of fly ash per cubic yard |
| silica_fume_C = float(input("Enter weight of silica fume per yard C: ")) | Defines a user input for the weight of fume silica per cubic yard |
| other_scm_D = float(input("Enter weight of SCM per yard D: ")) | Defines a user input for the weight of other cementitious material per cubic yard |
| wc_ratio_E = float(input("Enter target water/cement ratio E: ")) | Defines a user input for the water-to-cement ratio |
| air_content_F = float(input("Enter target % air content F: ")) | Defines a user input for the air content |
| percent_fine_agg_G = float(input("Enter target % fine agg G: ")) | Defines a user input for the percentage of fine aggregate |
| percent_coarse_agg_H = float(input("Enter target % coarse agg H: ")) | Defines a user input for the percentage of coarse aggregate |
| percent_other_agg_I = float(input("Enter target % other agg I: ")) | Defines a user input for the percentage of other aggregate |
| sg_cement_J = float(input("Enter sg of cement J: ")) | Defines a user input for the specific gravity of the portland cement |
| sg_fly_ash_K = float(input("Enter sg of fly ash K: ")) | Defines a user input for the specific gravity of the fly ash |
| sg_silica_fume_L = float(input("Enter sg of silica fume L: ")) | Defines a user input for the specific gravity of the fume silica |
| sg_other_scm_M = float(input("Enter sg of other scm M: ")) | Defines a user input for the specific gravity of the other cementitious material |
| sg_fine_agg_N = float(input("Enter sg of fine agg N: ")) | Defines a user input for the specific gravity of the fine aggregate |
| sg_coarse_agg_O = float(input("Enter sg of coarse agg O: ")) | Defines a user input for the specific gravity of the coarse aggregate |
| sg_other_agg_P = float(input("Enter sg of other agg P: ")) | Defines a user input for the specific gravity of the other aggregate |
| Q = water_weight_calc(cement_A,fly_ash_B,silica_fume_C,other_scm_D,wc_ratio_E) | Using the previously defined function and user inputs, calculates the required weight of water for the specified mix |
| R = calc_cem_vol(cement_A,sg_cement_J) | Using the previously defined function and user inputs, calculates the required weight of portland cement for the specified mix |
| S = calc_fly_ash_vol(fly_ash_B,sg_fly_ash_K) | Using the previously defined function and user inputs, calculates the required weight of fly ash for the specified mix |
| T = calc_silica_fume_vol(silica_fume_C,sg_silica_fume_L) | Using the previously defined function and user inputs, calculates the required weight of fume silica for the specified mix |
| U = calc_scm_vol(other_scm_D,sg_other_scm_M) | Using the previously defined function and user inputs, calculates the required weight of other cementitious material for the specified mix |
| V = calc_air_vol(air_content_F) | Using the previously defined function and user inputs, calculates the required volume of air for the specified mix |
| W = calc_water_vol(Q) | Using the previously defined function and user inputs, calculates the required volume of water for the specified mix |
| X = calc_agg_vol(R,S,T,U,V,W) | Using the previously defined function and user inputs, calculates the total aggregate volume for the specified mix |
| Y = calc_fine_agg_weight_Y(percent_fine_agg_G,sg_fine_agg_N,X) | Using the previously defined function and user inputs, calculates the required weight of fine aggregate for the specified mix |
| Z = calc_coarse_agg_weight_Z(percent_coarse_agg_H,sg_coarse_agg_O,X) | Using the previously defined function and user inputs, calculates the required weight of coarse aggregate for the specified mix |
| AA = calc_other_agg_weight_AA(W,percent_other_agg_I,sg_other_agg_P,X) | Using the previously defined function and user inputs, calculates the required weight of other aggregate for the specified mix |
| print("\n----------------------------------------------") print(" NDOT Concrete Mix Design – Weight Summary") print(" (1 Cubic Yard of Concrete)") print("----------------------------------------------") | Prints header for final table display of the required components of the specified concrete |
| print(f"Project Number: {project_number}") | Prints the specified project number |
| print(f"Class of Concrete: {concrete_class}") | Prints the specified concrete class |
| print("") | Prints an empty row in the table |
| print(f"Cement (A): {cement_A:5.1f} lb") | Prints the required weight of portland cement for the specified mix |
| print(f"Fly Ash (B): {fly_ash_B:5.1f} lb") | Prints the required weight of fly ash for the specified mix |
| print(f"Silica Fume (C): {silica_fume_C:5.1f} lb") | Prints the required weight of fume silica for the specified mix |
| print(f"Other SCM (D): {other_scm_D:5.1f} lb") | Prints the required weight of other cementitious material for the specified mix |
| print("") | Prints an empty row in the table |
| print(f"Fine Aggregate (Y): {Y:5.0f} lb") | Prints the required weight of fine aggregate for the specified mix |
| print(f"Coarse Aggregate (Z): {Z:5.0f} lb") | Prints the required weight of coarse aggregate for the specified mix |
| print(f"Other Aggregate (AA): {AA:5.0f} lb") | Prints the required weight of other aggregate for the specified mix |
| print("") | Prints an empty row in the table |
| print(f"Water (0): {Q:5.0f} lb") | Prints the required weight of water for the specified mix |
| print("----------------------------------------------") print("End of Mix Design Summary") | Prints footer for final table display of the required components of the specified concrete |