CIVE202 Project 2

NDOT Concrete Mix Design

Chris Hanson, Owen Schafer, Johnathan Sevick, Nietal Al Nahmo

- Introduction

In this project, we were tasked with turning the Nebraska Department of Transportation (NDOT)  concrete mix design Excel workbook into a Python script. The main objective is to turn the logic of the Excel workbook into an automatic Python script. Users should be able to use the script to input design parameters and material properties, and get a weight chart for one cubic yard of concrete. This project not only aimed to recreate the logic of the Excel workbook, but also improve reliability. Moving to a Python-based script makes the logic more transparent and repeatable.

- Methods

The NDOT Excel workbook logic was replicated using a series of functions in Python. The first function defined was to calculate water weight (Q). Adding up the weights of cement, fly ash, silica fume, and supplementary cementitious materials (SCMs) gives the total amount of cementitious material in our mix. This is then multiplied by the water-cement ratio to obtain the total weight of water needed in the mix. The formula is Q = (A + B + C+ D)  * E, where A is the weight of cement, B is the weight of fly ash, C is the weight of fly ash, D is the weight of SCMs, and E is the water-cement ratio.

The next set of functions calculates the total volume of each cementitious material. Each cementitious material’s volume can be calculated using the relation, Volume = W / (g * 62.2), where W is the weight of the cementitious material, g is the specific gravity of the material, and 62.2 is the unit weight of water in pounds per cubic foot. This formula can calculate the volume of cement (R), fly ash (S), silica fume (T), and SCMs (U).

Next, functions are defined to calculate the volume of air (V) and water (W) in the mix. Air volume in the mix is V = (F / 100) * 27, where F is the target percent of air in the mix, 27 is a conversion factor between cubic feet and cubic yards, and V is the total volume of air in the mix. Water volume is calculated by dividing the weight of water by the unit weight of water.

The last calculation needed is to find the weights of the aggregates. The total volume of the aggregates (X) is calculated as an intermediate step, X = 27 - (R + S + T + U + V + W), where R, S, T, and U are the volumes of cementitious materials, V is the volume of air, and W is the volume of water. The total volume of aggregates is then used to find the weights needed for each aggregate. For fine aggregate weight (Y), Y = 62.4 * (G / 100) * N * X, where G is the target percent of fine aggregate, and N is the specific gravity of fine aggregates. This same formula can be used to find the coarse aggregate weight (H) and other aggregate weight (I) by substituting the target percent and specific gravity.

Now that all necessary functions have been defined, all that is left is to take a user input, run the function, and print the outputs to the user. First, user input is taken. The user inputs values for project number, concrete class, cement weight, fly ash weight, silica fume weight, SCM weight, water-cement ratio, target air percent, target percent for each aggregate, and specific gravity for each material. Next, each function is run to calculate values. Finally, the outputs are printed out for the user. Weights for each material are printed for the user to examine.

- Results and Discussion

Mix 1: 47B-3500

Mix 47B-3500 is a commonly used mix for state highways.

Mix Specifications:

cement_A = 480

fly_ash_B = 98

silica_fume_C =  0

other_scm_D = 0

wc_ratio_E = 0.48

air_content_F = 6.25

percent_fine_agg_G = 70

percent_coarse_agg_H =  30

percent_other_agg_I = 0

sg_cement_J = 3.15

sg_fly_ash_K = 2.4

sg_silica_fume_L =2.2

sg_other_scm_M = 0

sg_fine_agg_N = 2.60

sg_coarse_agg_O = 2.65

sg_other_agg_P = 0

Calculated Values:

Fine Aggregates = 2018lbs

Coarse Aggregates = 882lbs

Other Aggregates = 0lbs

Water = 277lbs

Mix 2: 47BD-4000

Mix 47BD-4000 is commonly used for bridge decks.

Mix Specifications:

cement_A = 560

fly_ash_B = 113

silica_fume_C =  0.75

other_scm_D = 0

wc_ratio_E = 0.42

air_content_F = 6.25

percent_fine_agg_G = 70

percent_coarse_agg_H =  30

percent_other_agg_I = 0

sg_cement_J = 3.15

sg_fly_ash_K = 2.4

sg_silica_fume_L = 2.2

sg_other_scm_M = 0

sg_fine_agg_N = 2.60

sg_coarse_agg_O = 2.65

sg_other_agg_P = 0

Calculated Values:

Fine Aggregate = 1950lbs

Coarse Aggregate = 852lbs

Other Aggregate = 0lbs

Water = 283lbs

Mix 3: SF-3500

Mix SF-3500 is an overlay mix used to rehabilitate existing concrete.

Mix Specifications:

cement_A = 564

fly_ash_B = 2.4

silica_fume_C =  2.2

other_scm_D = 0

wc_ratio_E = 0.36

air_content_F = 5.5

percent_fine_agg_G = 50

percent_coarse_agg_H = 50

percent_other_agg_I = 0

sg_cement_J = 3.15

sg_fly_ash_K = 2.4

sg_silica_fume_L =2.25

sg_other_scm_M = 0

sg_fine_agg_N = 2.6

sg_coarse_agg_O = 2.65

sg_other_agg_P = 0

Calculated Values:

Fine Aggregate = 1547lbs

Coarse Aggregate = 1577lbs

Other Aggregate = 0lbs

Water = 212lbs

Mix 4: PR1-3500

Mix PR1-3500 is primarily used for the repair of concrete slabs that show cracks, corner breaks, or shattering.

Mix Specifications:

cement_A = 752

fly_ash_B = 0

silica_fume_C =  0

other_scm_D = 0

wc_ratio_E = 0.36

air_content_F = 5.5

percent_fine_agg_G = 70

percent_coarse_agg_H = 30

percent_other_agg_I = 0

sg_cement_J = 3.15

sg_fly_ash_K = 2.4

sg_silica_fume_L =2.2

sg_other_scm_M = 0

sg_fine_agg_N = 2.6

sg_coarse_agg_O = 2.65

sg_other_agg_P = 0

Calculated Values:

Fine Aggregates = 1970lbs

Coarse Aggregate = 861lbs

Other Aggregate = 0lbs

Water = 271lbs

These four mixes have different specifications and properties, designed for their specific purpose. For example, the bridge deck mix has a lower water-cement ratio than the highway mix. The bridge mix needs to be firmer than the highway mix. The bridge mix does have more total water, though, because it also has more total cementitious materials. The overlay mix and repair mix surprisingly have the least amount of water in them. They both have the lowest water-cement ratio while also having high amounts of cementitious materials. The amount of aggregate in each mix is also interesting. The highway and bridge mixes have high amounts of fine aggregate. They each have 70% by volume of fine aggregate. This makes the mix denser and therefore stronger, for what would be needed on a highway or bridge. The overlay mix uses the least amount of fine aggregates with a 50/50 by volume split between fine and coarse aggregates. Overall, each of these mixes has its own specifications and requirements suited to its usecase.

- References

Nebraska Department of Transportation. 2017. 1000-M20 – Portland Cement Concrete. Nebraska Department of Transportation. https://www.nebraskatransportation.org/ref-man/specbookm/1000/1000-m20.pdf.

Nebraska Department of Transportation. 2018. Pavement Design Manual. Materials and Research Division, Nebraska Department of Transportation. https://dot.nebraska.gov/media/jjwpyezr/pavdesignmanual.pdf

Nebraska Department of Transportation. 2017. Supplemental Specifications, Division 1000. Nebraska Department of Transportation. https://www.nebraskatransportation.org/ref-man/supspecem/SuppSpec-1000.pdf

Das, S., Morcous, G., and Hu, J. 2023. Low-Cement Concrete Mixture for Bridge Decks and Rails, Final Report SPR-FY22(001), Nebraska Department of Transportation and University of Nebraska-Lincoln. https://dot.nebraska.gov/media/jfbpdff4/morcous_ndot-final-report-09112013_pdfwcover.pdf

Gholami, S., Hu, J., and Kim, Y. R. 2019. Development of High-Performance Rapid Patching Materials for Pavement Repair. Research Report SPR-P1(18) M071, Nebraska Department of Transportation. https://dot.nebraska.gov/media/0ohcex3h/m071-development-of-high-performance-rapid-patching-materials.pdf

Engineering Reference. 2024. Cement Composition and Specific Gravity Data. Engineering Reference Database. https://www.sciencedirect.com/topics/engineering/cement-composition

Nebraska Department of Transportation. 2025. Bridge Design Manual: Chapters 1, 3, 5, 7, 9, 13 and Appendix C. Nebraska Department of Transportation. https://dot.nebraska.gov/media/4dapkdwu/2025-04-18_ndot-bdm-ch-1-3-5-7-9-13-app-c.pdf